Model PLOC: PLL Frequency Standard Noise Cleanup Oscillator

Features

- Output Frequencies up to 125 MHz

- Ultra-Low Phase Noise

- Self-Contained Frequency Standard

- Excellent Spectral Quality

- EMI Enclosure

- +5.0 VDC Operation/Low Power Design

Block Diagram

Description

The PLOC is a miniature assembly designed to phase-lock the internal OCXO to the input reference signal. The internal OCXO has exceptionally good phase noise performance, and when locked to a source with outstanding long-term stability, this module becomes the ideal frequency source for systems which require ultra-low phase noise. When the input is not present, the internal OCXO acts as the stand-alone precision frequency source.

PLOC Key Specifications

Output Frequency:	 	5 – 125 MHz

Spurious (max.):	 	-80 dBc

Harmonics (typ.):	 	-30 dBc

Output Power (min.): 	 	+13 dBm

Output Power Variation: 	±4 dB

External Reference:	 	5/10/100 MHz

Power Requirements: 		+15.0 VDC
Temp. Range:		  	-5°C to +65°C