Model PLOA: Analog Phase-Locked Looped Oscillator

Features

- Output Frequencies from 1 to 10 GHz

- Input Frequency:

  - Internal TCXO Reference

  - External Reference

- Low Phase Noise and Spurious

- Excellent Spectral Quality

- +5.0 VDC Operation/Low Power Design

Block Diagram

Description

Our PLOA line features a single Analog PLL design that locks a Voltage Controlled Oscillator to an external/internal reference.  This Analog PLL design enables access to a fine precision array of frequencies compared to other PLL devices, giving these units excellent phase noise and spectral quality. However, the output frequency must be an integer factor of the input reference frequency.

PLOA Key Specifications

Output Frequency:	 	1 – 10 GHz

Spurious:		 	-60 dBc

Harmonics (typ.):	 	-30 dBc

Output Power (min.): 	 	+13 dBm

Output Power Variation: 	±3 dB

External Reference:	 	10 - 200 MHz

Internal Reference:	 	±0.5 PPM

Alarm: 			 	Open Collector/TTL

Power Requirements:	 	+5.0 VDC @ 250 mA

Temp. Range:		  	0°C to +65°C