Model SLSM5: Single Loop Synthesizer – Rev. M5

Features

- Output Frequencies up to 34 GHz

- Frequency Control via RS-485 Interface:

  - Multi-Drop Bus Configuration

  - Non-Volatile Memory

  - USB w/ Optional Adapter

- Input Frequency:

  - Internal TCXO Reference

  - External Reference

- Low Phase Noise and Spurious

- Excellent Spectral Quality

- +5.0 VDC Operation/Low Power Design

Block Diagram

Description

The SLSM5 Frequency Synthesizer utilizes the latest Fractional N (÷N) technology to realize a compact high performance frequency synthesizer. This SLSM5 is ideal for many applications ranging from test equipment to SATCOM applications. The SLSM5 is system ready with minimal design integration required. For your leisure, a custom GUI is included for “out-of-the-box” operation.

SLSM5 Key Specifications

Output Frequency:	 	0.1 – 34 GHz

Spurious:		 	-60 dBc

Harmonics (typ.):	 	-17 dBc

Output Power (min.): 	 	+13 dBm

Output Power Variation: 	±2 dB

External Reference:	 	10 MHz

Internal Reference:	 	±0.5 PPM

Alarm: 			 	Open Collector/TTL

Power Requirements:	 	+5.0 VDC @ 600 mA

Temp. Range:		  	0°C to +60°C