Model TLSD/E: Triple Loop Synthesizer Rev. D/E

Features

- Output Frequencies up to 34 GHz

- Frequency Control via RS-232/422/485:

  - Multi-Drop Bus Configuration

  - USB w/ Optional Adapter

- Input Frequency:

  - Internal TCXO Reference

  - External Reference

- Low Phase Noise and Spurious

- Excellent Spectral Quality

- EMI Enclosure

- +5.0 VDC Operation/Low Power Design

Block Diagram

Description

The TLSD and TLSE models are high performance frequency synthesizers ideal for many SATCOM, military, and communication applications. These units, requiring an input frequency of 5, 10, or 100 MHz, are configured to automatically clean-up the input signal, and when there is no input detected, these units will automatically switch to their internal TCXO/OCXO.

TLSD/E Key Specifications

Output Frequency:	 	1 – 34 GHz

Spurious:		 	> -60 dBc

Harmonics (typ.):	 	-20 dBc

Output Power (min.): 	 	+13 dBm

Output Power Variation: 	±4 dB

External Reference:	 	5/10/100 MHz

Internal Reference:	 	±0.5 PPM

Load VSWR (max.):		2:1

Acquisition Time (typ.)		100 ms

Alarm: 			 	Open Collector/TTL

Temp. Range:		  	-5°C to +65°C